# PW16 V6.20+ Up/Download PC Software Installation

###### WARNING (UPDATING EXISTING SOFTWARE)

If you are updating to this newer version of the PW16 PC software, the XLD16i.cf folder must also be replaced when the new software is installed. This will mean that all customer files (panel configurations for installed panels) will be lost. Because of this you MUST follow the instructions below before updating the software.

First make a copy of the XLD16i.cf folder in the following folder;

C:\Program Files\Upload Download\16 Zone Upload Download

All of the saved customer files are stored inside the XLD16i.cf folder. By making a copy, if there are any problems caused by updating the software you have the ability to re-install the original folder so you can recover the customer files. This is very important and must be done before proceeding further.

At this point you should run the export program that will take all of the existing customer files and export them as separate files that can be accessed by this new software. Refer to the XLD16Export.doc for more details.

Now you can remove the existing software. Follow the path below to remove the software;

Start - Programs - Upload Download - 16 Zone Upload Download – Remove XLD16.

This will un-install the old software.

Once this has been done, proceed with the new installation as detailed below, except when the software finishes loading it will say that “an existing XLD16i.cf folder already exists”, “would you like to replace it?”. At this point select “Yes” and it will update the XLD16i.cf folder for you.

INSTALLING THE NEW SOFTWARE

# INSTALLING THE SOFTWARE

If a previous version of the software is already installed it is important that you first remove the existing software before installing the latest version otherwise the update will not have all of the new features and options. Follow the instructions above to remove the existing software.

To install the software, insert the CD into the CD drive.

Next, select “Run” from the start menu.

Type in; D:\setup.exe (where “D” denotes the default CD drive, if the drive letter is different use the correct letter for that PC),

then select “OK”.

(The program will start the installation process).

When the installation options appear select the “No Questions asked Installation” button and the software will complete the installation.

If this is a first time installation the software will automatically create a new folder called “16 Zone Upload Download” and install itself in the following area on your computers “C:” drive;

C:\Program Files\Upload Download\16 Zone Upload Download

It will also create a new Icon on your Desktop called XLD16i.

If the software installation is upgrading an older version, when the software finishes loading it will say that “an existing XLD16i.cf folder already exists”, “would you like to replace it?”. At this point select “Yes” and it will update the XLD16i.cf folder for you

### LOGGING ON TO THE PROGRAM

## Double click on this Icon and the new program will run automatically.

## The default LOGIN Name and PASSWORD are listed below.

DEFAULT LOGIN NAME = “PW16”

DEFAULT PASSWORD  = “PW16”

#### CHANGING YOUR PASSWORD

To change this password select “File”, “Passwords” then “Edit Existing”.

Then in the “Login Name” window type “PW16” then press the “Enter” button.

You may now put in your new password then confirm it in the second entry. At this point you can go back to the login name and change it to your own name.

When finished, press “OK” to save the changes. You may also create new users and their individual passwords with limited access to various operations by not ticking any of the option boxes when setting them up.

#### BACKING UP YOUR DATABASE

NOTE: All panel database files (data from customer panels) are now stored as individual files under the new “Config” folder. You should regularly make a backup of the “Config” folder so that you have copies of the customer files in case of a computer crash or similar problem. The login names and passwords to gain access to the software are stored in the XLD16i.cf folder. This can be backed up if you wish by making a copy of that folder but it is not as crucial as backing up the customer files regularly.